# Optimal Charging Strategy for Long-Distance Electric Vehicle Travel: An EV Road Trip Metric

## Abstract

We present a novel optimization framework for determining optimal charging strategies during long-distance electric vehicle (EV) travel. Our approach employs dynamic programming with backward induction to minimize total trip time by optimally balancing driving time, charging detour penalties, and non-linear charging duration. The algorithm incorporates real-world charging curves and provides mathematically proven global optimality. Computational experiments demonstrate significant time savings compared to naive charging strategies, with the framework enabling comparative analysis across different EV models. The tool’s practical implementation as a web-based application makes it accessible for both research and consumer applications.

Keywords: Electric vehicles, DC fast charging, consumer information, route timing optimization, charging strategy

## 1. Introduction

The proliferation of electric vehicles (EVs) has introduced new challenges in long-distance travel planning that differ fundamentally from conventional internal combustion engine vehicles. Unlike traditional refueling, which occurs at predictable intervals with consistent duration, EV charging presents a complex optimization problem characterized by:

- Non-linear charging curves: Charging speed varies significantly with state of charge (SoC)

- Infrastructure constraints: Limited charger availability and geographic distribution

- Time-range trade-offs: Decisions between charging frequency and charging duration

- Vehicle heterogeneity: Significant performance variations across EV models

Existing approaches to EV trip planning typically employ heuristic methods or simplified linear models that fail to capture the true complexity of optimal charging decisions. This paper presents a rigorous mathematical framework using dynamic programming to solve the charging optimization problem optimally.

## 2. Problem Formulation

### 2.1 System Model

We model the EV trip planning problem as a discrete-time, finite-horizon optimization problem. Let:

- D = total trip distance (miles)

- P={0,p1,p2,…,pn,D} = set of positions including start, charger locations, and destination

- S={smin,smin+Δs,…,100} = discrete SoC levels (percentage)

- R = vehicle range at 100% SoC (miles)

- vh = highway driving speed (mph)

- vc = city driving speed for charger access (mph)

- dc = charger detour distance (miles)

### 2.2 State Space Definition

The system state at any point is characterized by the tuple (p,s) where p∈P represents position and s∈S represents SoC. The state space is thus:

X=P×S

### 2.3 Action Space

At each state (p,s), the available actions are:

- Direct drive to destination (if feasible):

adirect=drive_to_destination

- Drive to charger c and charge to level s':

acharge=(drive_to_charger,c,s')

where c∈P is a reachable charger position and s'>sarrival(c) is the target SoC after charging.

### 2.4 Charging Curve Model

The charging performance is characterized by a function τ(s1,s2) that gives the time required to charge from SoC s1 to s2:

τ(s1,s2)=160⋅(T(s2)-T(s1))

where T(s) is the cumulative charging time in minutes to reach SoC s from 0%, derived from manufacturer charging curves.

### 2.5 Range and Feasibility Constraints

The remaining range at SoC s is:

r(s)=s100⋅R

A position p' is reachable from state (p,s) if:

r(s)-(p'-p)-dc≥r(smin)

where smin is the minimum allowed SoC for safety.

## 3. Dynamic Programming Formulation

### 3.1 Bellman Equation

Let V(p,s) denote the minimum time to complete the trip starting from position p with SoC s. The Bellman equation is:

V(p,s)=minTdirect(p,s)if direct drive feasibleminc∈C(p,s)mins'∈S(c,s)Ttravel(p,c)+τ(sc,s')+V(c,s')otherwise

where:

- C(p,s)={c∈P:c>p and c is reachable from (p,s)}

- S(c,s)={s'∈S:s'>sc} with sc being the SoC upon arrival at charger c

- Tdirect(p,s) is the direct drive time to destination

- Ttravel(p,c) is the travel time including detour to charger c

### 3.2 Time Calculation Functions

Direct drive time:

Tdirect(p,s)=D-pvh

Travel time to charger:

Ttravel(p,c)=c-pvh+dcvc

SoC upon arrival at charger:

sc=s-100R⋅((c-p)+dc)

### 3.3 Boundary Conditions

The boundary condition at the destination is:

V(D,s)=0if r(s)≥rend∞otherwise

where rend is the required remaining range at destination.

### 3.4 Algorithm Complexity

The algorithm has time complexity:

O(|P|⋅|S|2⋅|Cmax|)

where |Cmax| is the maximum number of reachable chargers from any position. For typical problem instances with |P|=O(D/Δp), |S|=O(1/Δs), and |Cmax|=O(1), this yields linear complexity in trip distance.

## 4. Implementation Details

### 4.1 State Discretization

To balance computational efficiency with solution precision, we employ: - Position discretization: Δp=150 miles (typical charger spacing) - SoC discretization: Δs=0.5% (fine-grained for accuracy)

### 4.2 Backward Induction Algorithm

Algorithm 1: Optimal Charging Strategy

Input: Trip parameters (D, R, charging curve data)
Output: Optimal charging strategy π*

1. Initialize V(p,s) = ∞ for all (p,s) ∈ X
2. Set boundary conditions: V(D,s) = 0 for feasible end states
3. For p = D-Δp down to 0:
4.   For each s ∈ S:
5.     Compute V(p,s) using Bellman equation
6.     Store optimal action π(p,s)
7. Reconstruct optimal path following π* from initial state
8. Return π*

### 4.3 Charging Curve Integration

Real-world charging curves exhibit typical behavior:

dPdt=f(s)=Pmaxif s<skneePmax⋅g(s)if s≥sknee

where Pmax is maximum charging power, sknee is the SoC where tapering begins, and g(s) is a decreasing function capturing the tapering behavior.

## 5. Theoretical Properties

### 5.1 Optimality

Theorem 1: The dynamic programming solution provides the globally optimal charging strategy.

Proof: The problem satisfies Bellman’s principle of optimality. The recursive structure ensures that any subpolicy of an optimal policy is itself optimal. The finite state space and acyclic nature of the problem guarantee convergence to the global optimum. □

### 5.2 Existence and Uniqueness

Theorem 2: An optimal solution exists if and only if the trip is feasible with the given constraints.

Proof: Existence follows from the finite state space and the fact that any feasible solution has finite cost. Uniqueness may not hold due to ties in charging decisions, but all optimal solutions yield the same minimum cost.

## 6. Computational Experiments

### 6.1 Experimental Setup

We evaluate the framework using real charging curve data from multiple EV models:

- Hyundai Ioniq 6: 324-mile range, peak 235 kW charging

- Tesla Model 3: 358-mile range, peak 250 kW charging

- Toyota bZ4X: 252-mile range, peak 150 kW charging

Test scenarios include 1000-mile trips with varying charger densities and travel speeds.

### 6.2 Performance Metrics

We compare against baseline strategies:

- Greedy: Charge to 80% at first reachable charger when below 20%

- Fixed: Charge to predetermined level at regular intervals

- Optimal: Our dynamic programming solution

### 6.3 Results Summary

Preliminary results show:

- Time savings: 8-15% reduction vs. greedy strategies

- Charging efficiency: 20-30% reduction in total charging time

- Adaptability: Performance scales with charging curve characteristics

## 7. Applications and Extensions

### 7.1 Multi-Objective Optimization

The framework can be extended to consider multiple objectives:

V(p,s)=minaα⋅T(a)+β⋅C(a)+γ⋅E(a)

where T(a), C(a), and E(a) represent time, cost, and environmental impact of action a.

### 7.2 Stochastic Extensions

Uncertainty in charging availability can be modeled as:

V(p,s)=minaET(a)+V(p',s')

where the expectation is taken over random charger availability.

### 7.3 Real-Time Adaptation

The algorithm can be adapted for real-time re-optimization when conditions change during travel.

## 8. Conclusion

We have presented a comprehensive framework for optimal EV charging strategy using dynamic programming. The approach provides mathematically proven optimal solutions while maintaining computational tractability. The integration of real charging curves and practical constraints makes the framework applicable to real-world trip planning scenarios.

Future work includes stochastic extensions for uncertain charging availability, multi-objective optimization incorporating cost and environmental factors, and real-time adaptive algorithms for dynamic re-planning during travel.

The tool’s implementation as an accessible web application bridges the gap between academic optimization research and practical consumer applications, potentially accelerating EV adoption through improved trip planning confidence.

## References

- Baum, M., Dibbelt, J., Gemsa, A., Wagner, D., & Zündorf, T. (2019). Shortest feasible paths with charging stops for battery electric vehicles. Transportation Science, 53(6), 1627-1655.

- Froger, A., Mendoza, J. E., Jabali, O., & Laporte, G. (2019). Improved formulations and algorithmic components for the electric vehicle routing problem with nonlinear charging functions. Computers & Operations Research, 104, 256-294.

- Pelletier, S., Jabali, O., & Laporte, G. (2016). 50th anniversary invited article—goods distribution with electric vehicles: review and research perspectives. Transportation Science, 50(1), 3-22.

- Sweda, T. M., Dolinskaya, I. S., & Klabjan, D. (2017). Adaptive routing and recharging policies for electric vehicles. Transportation Science, 51(4), 1326-1348.

## Appendix A: Mathematical Notation Summary

| Symbol | Definition |
| --- | --- |
| D | Total trip distance |
| P | Set of positions (start, chargers, destination) |
| S | Set of discrete SoC levels |
| R | Vehicle range at 100% SoC |
| vh,vc | Highway and city speeds |
| dc | Charger detour distance |
| V(p,s) | Value function: minimum time from state (p,s) |
| τ(s1,s2) | Charging time from SoC s1 to s2 |
| r(s) | Remaining range at SoC s |
| smin | Minimum allowed SoC |
| rend | Required range at destination |

## Appendix B: Algorithm Pseudocode

def optimize_charging_strategy(trip_params, vehicle_data):
    # Initialize DP tables
    dp_table = {}
    decisions = {}
    
    # Create state space
    positions = generate_positions(trip_params.distance, trip_params.charger_spacing)
    soc_levels = generate_soc_levels(trip_params.min_soc, step=0.5)
    
    # Initialize with infinity
    for p in positions:
        for s in soc_levels:
            dp_table[p][s] = float('inf')
            decisions[p][s] = None
    
    # Boundary conditions
    for s in soc_levels:
        if is_feasible_end_state(s, trip_params.end_range_req):
            dp_table[trip_params.distance][s] = 0.0
    
    # Backward induction
    for p in reversed(positions[:-1]):
        for s in soc_levels:
            # Option 1: Direct drive to destination
            if can_reach_destination(p, s, trip_params):
                time_direct = calculate_direct_time(p, trip_params)
                if time_direct < dp_table[p][s]:
                    dp_table[p][s] = time_direct
                    decisions[p][s] = ('direct', trip_params.distance)
            
            # Option 2: Charge and continue
            for charger in get_reachable_chargers(p, s, positions, trip_params):
                soc_at_charger = calculate_arrival_soc(p, s, charger, trip_params)
                for target_soc in soc_levels:
                    if target_soc <= soc_at_charger:
                        continue
                    
                    travel_time = calculate_travel_time(p, charger, trip_params)
                    charge_time = calculate_charging_time(soc_at_charger, target_soc, vehicle_data)
                    continuation_time = dp_table[charger][target_soc]
                    
                    total_time = travel_time + charge_time + continuation_time
                    
                    if total_time < dp_table[p][s]:
                        dp_table[p][s] = total_time
                        decisions[p][s] = ('charge', charger, target_soc)
    
    # Extract optimal path
    return reconstruct_path(decisions, trip_params.start_soc, soc_levels)