# EV Charging Curve Excel-to-XML Converter

Aaron Sobel, EPA

August 5, 2025

# Overview

This document explains the logic and methodology used by the EV Charging Curve Excel-to-XML Converter script. The script processes Argonne National Lab’s EV charging curve Excel spreadsheets and outputs a standardized XML format for use in the EV Road Trip Metric tool and other purposes.

# Python File Information

File Name: excel2xml.py

The following python libraries are required to be installed:

- pip install pandas numpy openpyxl

# Workflow

- Prepare Input Data: Place spreadsheets in the inputFiles folder

- Run Script: Run in Terminal: “python3 excel2xml.py”

- File Selection: Lists available Excel files in the inputFiles folder and prompts the user to select one.

- User Prompts: Asks the user to confirm interpolation and provide vehicle metadata (name, range).

- Data Extraction: Reads the spreadsheet, extracting columns for SOC, power, time, and energy.

- Data Processing: Ensures a complete 0–100% SOC curve, filling in missing values by interpolation and, if needed, extrapolation.

- XML Output: Outputs an XML file saved in the outputFiles folder.

# Data Handling: Interpolation and Extrapolation

## State of Charge (SOC)

- Input: May be incomplete (e.g., starts at 10%, ends at 90%).

- Output: Always produces a grid of integer SOC values from 0 to 100 (101 points).

- Method:

  - If the input SOC range does not cover 0–100%, the program fills in the missing ends.

  - For intermediate values, it uses linear interpolation.

## Time (minutes)

- Input: Read from the spreadsheet, converted from hh:mm:ss to minutes.

- Interpolation:

  - Uses linear interpolation for missing intermediate SOCs.

- Extrapolation:

  - Lower end:

    - For each extrapolated step, time increment is calculated as dE / start_power * 60 (minutes), where start_power is the average power of the first measured segment.

    - Cumulative sum gives the total time for each extrapolated SOC.

    - All measured times are shifted up by the total extrapolated time.

  - Upper end:

    - Extends time linearly using the last measured time increment.

## Energy (kWh)

- Input: Read from the spreadsheet, converted from Wh to kWh.

- Interpolation:

  - Uses linear interpolation for missing intermediate SOCs.

- Extrapolation:

  - Lower end:

    - Estimates total battery capacity by assuming the measured energy corresponds to the measured SOC range. If there are values for 80 out of 100 SOC data points, we know how much energy represents 80% of the battery capacity and can estimate how much energy needs to be extrapolated for between 0-10% SOC.

    - Extrapolates energy linearly from 0 up to the first measured value.

    - All measured energy values are shifted up by the extrapolated amount.

  - Upper end:

    - Extends energy linearly using the last measured increment.

## Power (kW)

- Input: Read from the spreadsheet, converted from W to kW.

- Interpolation:

  - For missing intermediate SOCs, uses linear interpolation to estimate power at each integer SOC.

- Extrapolation:

  - Lower end (below first measured SOC):

    - Power is calculated for each extrapolated step as the energy increment divided by the time increment for that step:

    - power = dE / (dt/60) (kWh per hour = kW).

    - Difference in energy over difference in time divided by 60 minutes.

    - The time increment for each step is based on the template power from the first measured segment.

    - Any division by zero or invalid values are set to zero.

    - This generates a flat power curve averaged based on the estimated energy and time.

  - Upper end (above last measured SOC):

    - Uses the last measured power value for all extrapolated steps.

# Summary Table

| Category | Interpolation | Extrapolation (Lower) | Extrapolation (Upper) |
| --- | --- | --- | --- |
| SOC | Linear grid 0–100 | Fills missing below first measured | Fills missing above last measured |
| Power | Linear | Calculated as dE/dt for each step | Uses last measured |
| Time | Linear | dE/template_power × 60, cumulative | Linear extension |
| Energy | Linear | Linear from 0, shifts all up | Linear extension |

# User Prompts

- The script asks for permission before extrapolating outside the measured SOC range.

- It also asks if interpolation is allowed for missing intermediate values.

# Output

- The XML file contains 101 <point> elements (SOC 0–100), each with interpolated/extrapolated values for power, time, and energy.