# EV Road Trip Calculator: Short Description

This tool provides a standardized way to compare EV performance by simulating how long different vehicles take to complete a hypothetical long-distance trip. Instead of looking at range, efficiency, and charging speed separately, it combines these factors into one clear metric: time to travel 1,000 miles.

The tool finds the fastest route for each EV using a dynamic programming approach, which works backwards from the destination to evaluate all possible scenarios before then working forward to determine the optimal path.

The algorithm divides the trip into key points: the starting location, potential charging stations, and the destination. At each location, it considers various battery levels you might have upon arrival.

Starting backwards from the destination, the system calculates the minimum time needed to complete the trip from each possible state (location and battery level). It evaluates all options from each state - driving directly to the destination, stopping at reachable chargers, or charging to different levels before continuing.

After this backwards analysis, the tool reconstructs the optimal path by starting from the trip beginning and following the best decisions it has already calculated. This produces the itinerary: which charging stations to use, how much to charge at each stop, and the sequence that minimizes total travel time.

The algorithm accounts for constraints including the vehicle's charging curve, battery level limits, charging station locations, detour distances, and destination range requirements.